Computer Networks

LAB Manual 04

Objectives:

4.1	Dynamic Host Configuration Protocol (DHCP) Configuration by using a Server and a Cisco Switch through Packet Tracer

4.2 Setting up FTP Server in Windows 10 and Packet Capturing

4.1	Dynamic Host Configuration Protocol (DHCP) Configuration by using a Server and a Cisco Switch through Packet Tracer

PREREQUISITES: Laptop / PC, Cisco Packet Tracer Simulation Software.

Procedure / Steps

Through Cisco Packet Tracer:

Step 1: Open Cisco Packet Tracer Application and login with your netacad credentials. If you don’t have netacad account, then you must create it first.

Step 2:  Go to network devices option at the left bottom of packet tracer screen and choose Hub as a network device.

Step 3:  Go to end devices option at the left bottom of packet tracer screen and choose Five Devices (PCs) from there.

Step 4:  Choose the right cable connection (Straight Through) as you are connecting two different devices this time and appropriate port in PC and network hub as well.

Step 5:  Add a server device from “End Devices” option where you have selected the PC and connect it with the hub like you connected the PCs with it.

Step 6:  Double click on the Server and go to interface setting and assign the following IP address to the server.

Step 7:  Go to the services and then click on the “DHCP” option at left.

Step 8:  Configure the following DHCP setting and then click on the “Save” button.

Step 9:  Double click on PC and then go Desktop and click on IP configuration.

Step 10:  Click on “DHCP”, wait for some moment, the DHCP server will assign the IP address automatically to the PC from the pool that configured in Step 8.

Note: Repeat the Step 10 for other PCs as well.

Through Cisco Switch:

Step 1: Open Cisco Packet Tracer Application and login with your netacad credentials. If you don’t have netacad account, then you must create it first.

Step 2:  Go to network devices option at the left bottom of packet tracer screen and choose Switch as a network device.

Step 3:  Go to end devices option at the left bottom of packet tracer screen and choose Five Devices (PCs) from there.

Step 4:  Choose the right cable connection (Straight Through) as you are connecting two different devices this time and appropriate port in PC and network hub as well.

Step 5:  Double click on the Switch and go to CLI tab and click on it.

Step 5:  Double click and press “Enter” in the “IOS Command Line Interface area and do the following configuration.

Switch>enable

Switch#configure terminal

Switch(config)#interface vlan 1

Switch(config-if)#ip address 192.168.1.1 255.255.255.0

Switch(config-if)#no shutdown

Switch(config-if)#exit

Switch(config)#ip dhcp pool CN-LAB

Switch(dhcp-config)#network 192.168.1.100 255.255.255.0

Switch(dhcp-config)#default-router 192.168.1.1

Switch(dhcp-config)#dns-server 192.168.1.1

Switch(dhcp-config)#exit

Switch(config)#ip dhcp excluded-address 192.168.1.1

Switch(config)#exit

Switch#copy running-config startup-config

Step 6:  Double click on PC and then go Desktop and click on IP configuration.

Step 7:  Click on “DHCP”, wait for some moment, the DHCP server will assign the IP address automatically to the PC from the pool that configured in Step 7.

Note: Repeat the Step 7 for other PCs as well.

4.2 Setting up FTP Server in Windows 10 and Packet Capturing

Procedure / Steps

5.1.1 Setting Up FTP Server in Windows 10:

Step 1:  Create a new folder with “FTP” name on the desktop.

Step 2:  Go to the folder properties, then security tab and set the permission for FTP folder. In below snapshot I have given full control to a user “Hafiz Adil Rasheed”. You need to set it accordingly.

Step 3:  The third step is open control panel of windows 10 and then “Programs and Features.” And then click on “Turn windows features on or off.”

Step 4:  In “Windows Features” windows select and expand “Internet Information Services”. Check the “FTP Extensibility” and “FTP Services” and then click on OK button.

Step 5:  Go to control panel of windows 10 and then click on “Administrative Tools”.

Step 6:  In Administrative Tools” doble click on Internet Information Services (IIS) Manager.

Step 6:  On the left under “Connections”, expand your PC name.

Step 7:  Right click on the “Sites” tab in Internet Information Services (IIS) Manager and then click on “Add FTP Site..”

Step 7:  Enter FTP Site name as “CN-LAB” and give the physical path of “FTP” folder created earlier and then click on Next.

Step 8:  In Binding and SSL setting, do the following setting demonstrated below.

Step 9:  In “Authentication and Authorization Information” do the following setting demonstrated below and click on Finish button.

Step 10:  The FTP server is now successfully has been added.

Step 11:  Check the FTP Services with the help of a third-party FTP client. We are using “Filezilla Client” in this lab.

Step 12:  Open the “Filezilla Client” and enter the following Host (IP address of FTP Server) , Username (Authorized User defined in FTP Server), password and port (21) and then click on “Quickconnect”

Step 13:  The connection has now been established between FTP Server and FTP client. Now you can upload and download data.